ANNUAL REPORT

| Drinking-Water System Number: | 2600002460 |
| --- | --- |
| Drinking-Water System Name: | Cambridge Drinking Water System |
| Drinking-Water System Owner: | The Corporation of the City of Cambridge |
| Drinking-Water System Category: | Large Municipal Residential |
| Period being reported: | January 1 to December 31, 2025 |

| Complete if your Category is Large Municipal Residential or Small Municipal Residential Does your Drinking-Water System serve more than 10,000 people? Yes [x] No [ ] Is your annual report available to the public at no charge on a web site on the Internet? Yes [x] No [ ] Location where Summary Report required under O. Reg. 170/03 Schedule 22 will be available for inspection. A copy may be obtained from the City of Cambridge Clerk’s Department 519-623-1340 | Complete for all other Categories. Number of Designated Facilities served: N/A Did you provide a copy of your annual report to all Designated Facilities you serve? Yes [ ] No [ ] Number of Interested Authorities you report to: N/A Did you provide a copy of your annual report to all Interested Authorities you report to for each Designated Facility? Yes [ ] No [ ] |
| --- | --- |

Note: For the following tables below, additional rows or columns may be added, or an appendix may be attached to the report

List all Drinking-Water Systems (if any), which receive all of their drinking water from your system:

| Drinking Water System Name | Drinking Water System Number |
| --- | --- |
| Lloyd Brown Subdivision Distribution System | 260002759 |

Did you provide a copy of your annual report to all Drinking-Water System owners that are connected to you and to whom you provide all of its drinking water?

Yes [x]    No [  ]

Indicate how you notified system users that your annual report is available, and is free of charge.

[x] Public access/notice via the web

[x] Public access/notice via Government Office

[  ] Public access/notice via a newspaper

[x] Public access/notice via Public Request

[  ] Public access/notice via a Public Library

[x] Public access/notice via other method: Report provided to Cambridge City Council

Describe your Drinking-Water System

The Cambridge Distribution System services approximately 138,500 people via 44,729 service connections, which includes 42,607 private residences. There is an estimated 616.7 kilometers of water main located within Cambridge, of which the City owns 526.7 km, the Region of Waterloo owns 60.8 km, and there is 29.2 km of shared mains. There are 3,731 hydrants providing fire protection and 6,501 valves (including shared) of various sizes for controlling water flow. The distribution system is divided into six (6) individual pressure zones based on the water supply components and the varying elevations throughout the City. The water supply is provided by the Region.

List all water treatment chemicals used over this reporting period

N/A – See Region of Waterloo

Were any significant expenses incurred to?

[  ] Install required equipment

[  ] Repair required equipment

[  ] Replace required equipment

# Please provide a brief description and a breakdown of monetary expenses incurred

Water Connection Maintenance – $3,526,354

Watermain Maintenance – $1,542,751

Hydrant Maintenance - $ 682,723

*Numbers are approximate at time of report

Provide details on the notices submitted in accordance with subsection 18(1) of the Safe Drinking-Water Act or section 16-4 of Schedule 16 of O.Reg.170/03 and reported to Spills Action Centre

| Incident Date | Parameter | Result | Unit of Measure | Corrective Action | Corrective Action Date |
| --- | --- | --- | --- | --- | --- |
| 2025-02-03 (AWQI# 167325) | Low Chlorine | <0.05 mg/L Free Chlorine | mg/L | Flushed and resampled | 2025-02-03 |
| 2025-05-23 (AWQI# 168335) | Low Chlorine | <0.05 mg/L Free Chlorine | mg/L | Flushed and resampled | 2025-05-24 |
| 2025-07-10 (AWQI# 168945) | Total Coliforms | Present | Present/ Absent | Flushed and resampled | 2025-07-11 |
| 2025-08-28 (AWQI# 169628) | Low Chlorine | <0.25 mg/L Combined Chlorine | mg/L | Flushed and resampled | 2025-08-28 |
| 2025-08-28 (AWQI# 169629) | Low Chlorine | <0.25 mg/L Combined Chlorine | mg/L | Flushed and resampled | 2025-08-28 |
| 2025-09-02 (AWQI# 169710) | Low Chlorine | <0.25 mg/L Combined Chlorine | mg/L | Flushed and resampled | 2025-09-02 |
| 2025-09-25 (AWQI# 170116) | Total Coliforms | Present | Present/ Absent | Flushed and resampled | 2025-09-26 |
| 2025-10-23 (AWQI# 170493) | Low Chlorine | <0.25 mg/L Combined Chlorine | mg/L | Flushed and resampled | 2025-10-23 |
| 2025-10-30 (AWQI# 170592) | Low Chlorine | <0.25 mg/L Combined Chlorine | mg/L | Flushed and resampled | 2025-10-30 |

Microbiological testing done under the Schedule 10, 11 or 12 of Regulation 170/03, during this reporting period.

|  | Number of Samples | Range of E. Coli Or Fecal Results (min #)-(max #) | Range of Total Coliform Results (min #)-(max #) | Number Of HPC Samples | Range of HPC Results (min #)-(max #) |
| --- | --- | --- | --- | --- | --- |
| Distribution | 1621 | Absent, Present | Absent, Present | 908 (56% of samples) | 0 - >500 CFU/mL |

Operational testing done under Schedule 7, 8 or 9 of Regulation 170/03 during the period covered by this Annual Report.

|  | Number of Grab Samples | Range of Results (min #)-(max #) | Unit of Measure |
| --- | --- | --- | --- |
| Total Chlorine | 832 | 0.26 – 1.70 | mg/L |
| Combined Chlorine | 832 | 0.03 – 1.62 | mg/L |
| Free Chlorine | 832 | 0.01 – 1.52 | mg/L |

Note: For continuous monitors use 8760 as the number of samples

Summary of additional testing and sampling carried out in accordance with the requirement of an approval, order or other legal instrument.

| Date of legal instrument issued | Parameter | Date Sampled | Result | Unit of Measure |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

Summary of Inorganic parameters tested during this reporting period or the most recent sample results

| # Parameter | # Sample Date | Result Value | # Unit of Measure | # Exceedance |
| --- | --- | --- | --- | --- |
| ## Antimony | N/A |  |  |  |
| Arsenic |  |  |  |  |
| Barium |  |  |  |  |
| Boron |  |  |  |  |
| Cadmium |  |  |  |  |
| ## Chromium |  |  |  |  |
| Mercury |  |  |  |  |
| Selenium |  |  |  |  |
| Sodium |  |  |  |  |
| Uranium |  |  |  |  |
| Fluoride |  |  |  |  |
| Nitrate |  |  |  |  |
| Nitrate |  |  |  |  |

*only for drinking water systems testing under Schedule 15.2; this includes large municipal non-residential systems, small municipal non-residential systems, non-municipal seasonal residential systems, large non-municipal non-residential systems, and small non-municipal non-residential systems

Summary of lead testing under Schedule 15.1 during this reporting period

(applicable to the following drinking water systems; large municipal residential systems, small municipal residential systems, and non-municipal year-round residential systems)

| Location Type | Number of Samples | Range of Lead Results (min#) – (max #) | Unit of Measure | Number of Exceedances |
| --- | --- | --- | --- | --- |
| Alkalinity (Distribution System) | 44 | 264 - 353 | mg/L | n/a (not legislated) |
| pH (Distribution System, Residential, and Non-Residential | 154 | 6.47 – 7.71 | pH | n/a (not legislated) |
| Plumbing | 220 | <0.200 – 16.1 | µg/L | 2 |
| Distribution | 44 |  | µg/L | 0 |

Summary of Organic parameters sampled during this reporting period or the most recent sample results

| Parameter | # Sample Date | Result Value | # Unit of Measure | # Exceedance |
| --- | --- | --- | --- | --- |
| Alachlor |  |  |  |  |
| Atrazine + N-dealkylated metabolites |  |  |  |  |
| Azinphos-methyl |  |  |  |  |
| Benzene |  |  |  |  |
| Benzo(a)pyrene |  |  |  |  |
| Bromoxynil |  |  |  |  |
| Carbaryl |  |  |  |  |
| Carbofuran |  |  |  |  |
| Carbon Tetrachloride |  |  |  |  |
| Chlorpyrifos |  |  |  |  |
| Diazinon |  |  |  |  |
| Dicamba |  |  |  |  |
| 1,2-Dichlorobenzene |  |  |  |  |
| 1,4-Dichlorobenzene |  |  |  |  |
| 1,2-Dichloroethane |  |  |  |  |
| 1,1-Dichloroethylene (vinylidene chloride) |  |  |  |  |
| Dichloromethane |  |  |  |  |
| 2-4 Dichlorophenol |  |  |  |  |
| 2,4-Dichlorophenoxy acetic acid (2,4-D) |  |  |  |  |
| Diclofop-methyl |  |  |  |  |
| Dimethoate |  |  |  |  |
| Diquat |  |  |  |  |
| Diuron |  |  |  |  |
| Glyphosate |  |  |  |  |
| HAA (NOTE: show latest annual average) | 2025-02-10 2025-05-23 2025-08-14 2025-11-20 | 5.19 | µg/L | No |
| Malathion |  |  |  |  |
| 2-Methyl-4-chlorophenoxyacetic acid (MCPA) |  |  |  |  |
| Metolachlor |  |  |  |  |
| Metribuzin |  |  |  |  |
| Monochlorobenzene |  |  |  |  |
| Paraquat |  |  |  |  |
| Pentachlorophenol |  |  |  |  |
| Phorate |  |  |  |  |
| Picloram |  |  |  |  |
| Polychlorinated Biphenyls(PCB) |  |  |  |  |
| Prometryne |  |  |  |  |
| Simazine |  |  |  |  |
| Terbufos |  |  |  |  |
| Tetrachloroethylene |  |  |  |  |
| 2,3,4,6-Tetrachlorophenol |  |  |  |  |
| THM (NOTE: show latest annual average) | 2025-03-06 2025-05-23 2025-08-14 2025-11-13 | 42.95 | µg/L | No |
| Triallate |  |  |  |  |
| Trichloroethylene |  |  |  |  |
| 2,4,6-Trichlorophenol |  |  |  |  |
| Trifluralin |  |  |  |  |
| Vinyl Chloride |  |  |  |  |

List any Inorganic or Organic parameter(s) that exceeded half the standard prescribed in Schedule 2 of Ontario Drinking Water Quality Standards.

| Parameter | Result Value | Unit of Measure | Date of Sample |
| --- | --- | --- | --- |
| Lead (Residential) | 8.68 | µg/L | 2025-03.05 |
| Lead (Residential) | 5.42 | µg/L | 2025-03.12 |
| Lead (Residential) | 6.40 | µg/L | 2025-04-09 |
| Lead (Residential) | 5.96 | µg/L | 2025-06-18 |
| Lead (Residential) | 6.70 | µg/L | 2025-06-25 |
| Lead (Residential) | 7.23 | µg/L | 2025-07-23 |